CanHeader (the frame metadata) and CanPayload (the 8 raw data bytes)

module Drv {

struct CanHeader {

isExtended: bool    // standard (11-bit) or extended (29-bit) CAN ID

$id:        U32     // the actual CAN message ID

dataLength: U8      // how many bytes follow (0–8)

timestamp:  U16     // hardware RX timestamp

device:     U8      // which CAN controller (VA416x0 has CAN0) – What is this device ?

}

array CanPayload = [8] U8   // exactly 8 bytes, the raw CAN data field

}

CanMessage

port CanMessage(

ref header: CanHeader

ref data:   CanPayload

) -> ByteStreamStatus

CanFilter – Is this driver specific? Scope for UCOF ?

port CanFilter(

bankNum:        U8      @< Bank/Mailbox number

$id:            U32     @< Partial or full CAN id to match (11 or 29 bit)

mask:           U32     @< Filter mask fitting the id match pattern (11 or 29 bit)

isGlobal:       bool    @< Global filter flag

isExtended:     bool    @< Extended flag

$priority:      U8      @< Priority number (implementation dependent on CAN peripheral)

enable:         bool    @< Enable flag (false to disable filter on mailbox)

) -> bool                   @< Return success flag

CanDriver // what the HARDWARE SIDE provides

interface CanDriver {

guarded input port setRxFilter: CanFilter

guarded input port canRx: CanMessage    // read a frame from HW

guarded input port canTx: CanMessage    // write a frame to HW

output port ready: ByteStreamReady      // "I'm initialised"

}

CanManager // what the SOFTWARE SIDE needs

interface CanManager {

sync input port drvConnected: ByteStreamReady

output port setRxFilter: CanFilter

output port canRx: CanMessage

output port canTx: CanMessage

}